Literature Survey: Applications of AI/ML Algorithms in L1/L2 Triggered Mobility (LTM) and L3 Handover

### 1. Introduction

### The seamless and efficient management of mobility is very important in the current mobile networks especially while moving from one network generation to another for example from 5G to 6G. The Layer 1/Layer 2 Triggered Mobility (LTM) method was introduced in 3GPP Release 18 to enhance handover performance by using L1/L2 measurements instead of traditional Layer 3 (L3) signaling to reduce latency and signaling load [1]. However, L3 handover is still the dominant method of mobility management that relies on RRC messages to accomplish the process [2]. The integration of Artificial intelligence and Machine learning in both LTM and L3 handovers can be used to enhance network performance through adaptive, data-driven decision making [3].

### 2. Background on L1/L2 Triggered Mobility (LTM) and L3 Handover

#### 2.1. L1/L2 Triggered Mobility (LTM)

#### LTM uses lower layer measurements to manage mobility, and thus handovers are faster and more reliable than with conventional L3 based methods. The LTM process consists of three main steps:

#### • LTM Preparation: The UE sends an RRC measurement report to the gNB, which sets up a list of candidate cells for handover and sends candidate configurations to the UE [1].

#### • LTM Synchronization and Decoding: The UE turns on a TCI state for the candidate cell and obtains early TA (Timing Advance) to ensure uplink alignment before handover [2].

#### • LTM Execution: The UE sends an L1 measurement report to the gNR, which triggers the handover according to real-time conditions and finalizes the process with minimal service disruption [1].

#### 2.2. Layer 3 (L3) Handover with RRC Messages in 5G:

L3 handover is the conventional mobility management scheme in cellular networks based on RRC(Radio Resource Control)  for the purpose of handover. The main phases of L3 handover are as follows:

- Measurement  Reporting (RRC Measurement Report): The UE regularly sends measurement report on the signal strength and quality  to the serving gNB [2].

- Handover Decision (RRC Handover Preparation): The gNB analyzes the reports and establishes whether a handover is needed based on a predefined thresholds[3].

- Handover Execution (RRC Connection Reconfiguration): The source gNB selects the target cell, and the UE receives an RRC handover command before switching to the new cell [2]. Even  though L3 handover is more reliable than LTM, it entails higher signaling load and longer latency  as compared to LTM [1]. The application of AI/ML in the management of L3 handover comprises of identifying the best time to perform a handover, preventing unnecessary handovers and enhancing the performance of the network [4].

### 3. AI/ML Applications in Mobility Management

#### 3.1. AI/ML for LTM Optimization in 5G

AI/ML techniques are increasingly being used to optimize LTM handovers by predicting when and where handovers should occur using historical and real time data [3]. Supervised learning models like decision trees and deep neural networks can improve mobility prediction accuracy [4]. Reinforcement learning (RL) methods such as Deep Q-Networks (DQN) can adapt handover triggers dynamically based on real time network performance [5].

#### 3.2. AI/ML for L3 Handover Optimization in 5G

AI/ML methods are also implemented for conventional L3 handovers in 5G networks essentially in optimizing RRC-based handover procedures. These approaches enhance the prediction accuracy in handover decisions to avoid unnecessary handovers and to improve network stability [2]. ML models incorporate mobility patterns and signal strength predictions to enhance the efficiency of L3 handover and reduce the signaling overhead [5].

#### 3.3. AI in Synchronization and Random Access in 5G

Efficient synchronization is vital for both LTM and L3 handover. AI-based models can predict synchronization patterns and optimize access procedures.

- Primary Synchronization Signal (PSS): It is used for initial cell detection [1].

- Secondary Synchronization Signal (SSS): It is used for frame synchronization and PCI (Physical Cell Identity) determination [2].

- Physical Broadcast Channel (PBCH): It transmits essential system information, such as the Master Information Block (MIB) [3].

These synchronization mechanisms are crucial for AI-driven mobility decisions, allowing for faster and more efficient handover execution [4].

### 4. Key Research Challenges

- Frequent Handover Events: AI models must balance handover frequency to prevent against excessive switching between cells, which can cause service interruptions [5].

- Interference Management: LTM and L3 handovers must mitigate inter-cell interference, requiring AI-based optimization techniques [3].

- Real-Time Adaptability: AI models should be capable for making real-time mobility decisions under varying network conditions [4].

- Data Collection and Processing: AI/ML models require large, high-quality datasets for training, which can be challenging to obtain that in real-world deployments [5].

- Optimization of RRC Signaling in L3 Handover: AI-based models can enhance the decision-making process for when and how RRC signaling should be triggered to reduce latency and network congestion [2].

### 5. Future Directions

- AI Approaches: By combining both AI-driven decision-making with rule-based heuristics,  it can improve reliability [3].

- Federated Learning for Distributed Model Training: AI models can be trained across multiple network nodes while preserving the data privacy [5].

- Edge AI for Low-Latency Decisions: Deploying AI models at the network edge can reduce response times and enhance mobility efficiency [4].

- 6G Integration: AI/ML-driven mobility management will play a crucial role in 6G networks, requiring continued research and development [1].

### 6. Conclusion

### The integration of AI/ML into LTM and L3 handover mechanisms has shown that there  is a huge potential for increasing the network efficiency and reducing latency. Supervised and reinforcement learning models leverage AI driven predictions to optimize mobility management and can enable a seamless handover experience for users [3].  However, challenges like interference management, real time adaptability and data availability need to be solved to enable  the practical implementation of AI in mobility management [5]. Thus, the integration of AI/ML into  LTM and L3 handover mechanisms has the potential to enhance the network efficiency and reduce latency.  AI driven predictions support supervised and reinforcement learning models to optimize mobility management and can lead to a seamless  handover experience for users [3]. However, to practically implement AI in mobility management, issues such as  interference management, real time adaptability, and data availability must first be resolved [5].

### 7. References:

### [1]

# Bibliography

| [1] | M. a. G. X. a. J. R. Tayyab, "A survey on handover management: From LTE to NR," IEEE Access, vol. 7, pp. 118907--118930, 2019. |
| --- | --- |
| [2] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, 2024. |
| [3] | M. K. a. J. S. a. J. I. Dahouda, "Machine learning-based solutions for handover decisions in non-terrestrial networks," Electronics, vol. 12, p. 1759, 2023. |
| [4] | J. a. H. A. a. A. R. a. K. A. Tanveer, "An overview of reinforcement learning algorithms for handover management in 5G ultra-dense small cell networks," Applied Sciences, vol. 12, p. 426, 2022. |

| [1] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, 2024. |
| --- | --- |

[2] M. K. a. J. S. a. J. I. Dahouda, "Machine learning-based solutions for handover decisions in non-terrestrial networks," Electronics, vol. 12, p. 1759, 2023.

[3] J. a. H. A. a. A. R. a. K. A. Tanveer, "An overview of reinforcement learning algorithms for handover management in 5G ultra-dense small cell networks," Applied Sciences, vol. 12, p. 426, 2022.

### [2]

# Bibliography

[3]

# Bibliography

| [1] | M. a. G. X. a. J. R. Tayyab, "A survey on handover management: From LTE to NR," IEEE Access, vol. 7, pp. 118907--118930, 2019. |
| --- | --- |
| [2] | Z. a. B. N. a. M.-B. J. a. G. L. Ali, "Machine learning based handover management for improved QoE in LTE," in NOMS 2016-2016 IEEE/IFIP Network Operations and Management Symposium, 2016, pp. 794--798. |
| [3] | R. a. K. G. a. C. S. Karmakar, "Mobility management in 5G and beyond: A novel smart handover with adaptive time-to-trigger and hysteresis margin," IEEE Transactions on Mobile Computing, vol. 22, pp. 5995--6010, 2022. |
| [4] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, no. IEEE, 2024. |
| [5] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, 2024. |
| [6] | M. K. a. J. S. a. J. I. Dahouda, "Machine learning-based solutions for handover decisions in non-terrestrial networks," Electronics, vol. 12, p. 1759, 2023. |
| [7] | J. a. H. A. a. A. R. a. K. A. Tanveer, "An overview of reinforcement learning algorithms for handover management in 5G ultra-dense small cell networks," Applied Sciences, vol. 12, p. 426, 2022. |
| [8] | M. S. a. A. A. I. a. O. M. a. K. S. F. a. K. M. a. H. S. a. I. M. A. a. A. Q. H. Mollel, "A survey of machine learning applications to handover management in 5G and beyond," IEEE Access, vol. 9, pp. 45770--45802, 2021. |

# Bibliography

| [1] | M. a. G. X. a. J. R. Tayyab, "A survey on handover management: From LTE to NR," IEEE Access, vol. 7, pp. 118907--118930, 2019. |
| --- | --- |
| [2] | Z. a. B. N. a. M.-B. J. a. G. L. Ali, "Machine learning based handover management for improved QoE in LTE," in NOMS 2016-2016 IEEE/IFIP Network Operations and Management Symposium, 2016, pp. 794--798. |
| [3] | R. a. K. G. a. C. S. Karmakar, "Mobility management in 5G and beyond: A novel smart handover with adaptive time-to-trigger and hysteresis margin," IEEE Transactions on Mobile Computing, vol. 22, pp. 5995--6010, 2022. |
| [4] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, no. IEEE, 2024. |
| [5] | B. a. G. S. a. G. M. a. S. J. a. K. A. a. T. R. a. B. A. a. S. P. a. M. C. Khodapanah, "On L1/L2-Triggered Mobility in 3GPP Release 18 and Beyond," IEEE Access, 2024. |
| [6] | M. K. a. J. S. a. J. I. Dahouda, "Machine learning-based solutions for handover decisions in non-terrestrial networks," Electronics, vol. 12, p. 1759, 2023. |
| [7] | J. a. H. A. a. A. R. a. K. A. Tanveer, "An overview of reinforcement learning algorithms for handover management in 5G ultra-dense small cell networks," Applied Sciences, vol. 12, p. 426, 2022. |
| [8] | M. S. a. A. A. I. a. O. M. a. K. S. F. a. K. M. a. H. S. a. I. M. A. a. A. Q. H. Mollel, "A survey of machine learning applications to handover management in 5G and beyond," IEEE Access, vol. 9, pp. 45770--45802, 2021. |

[3]