#### Radio Protocol Architecture

The Radio Access Network (RAN) protocol architecture in cellular networks (e.g., 5G, LTE, NR) defines how data is transmitted, received, and processed in a structured layered model.

### 1. User Plane

The User Plane is responsible for transferring user data (e.g., video, voice, browsing data) between the User Equipment (UE) and gNB. It involves the following key protocols and layers:

- SDAP: It manages Quality of Service (QoS).

- PDCP: It handles IP header compression, ciphering, and reordering.

- RLC: It is responsible for segmentation and retransmissions.

- MAC: It Handles logical channel multiplexing and HARQ retransmissions.

- PHY: It manages signal modulation and transmission.

#### Security & Reliability Focus

- The encryption is handled through PDCP ciphering.

- The packet reordering is managed at the PDCP layer to maintain reliability.

#### AI Applications

- AI can optimize packet reordering, HARQ retransmissions, and duplicate detection in mobility scenarios to reduce packet loss.

### 2. Control Plane

The Control Plane handles signaling between the UE and gNB to manage network functions like mobility, authentication, and security. Key protocols include:

- RRC (Radio Resource Control) – Manages connection setup, bearer management, and paging.

- NAS (Non-Access Stratum) – Handles core network signaling.

- PDCP, RLC, MAC, and PHY layers – Support control signaling.

#### Security Focus

- The encryption and integrity protection are applied to RRC (Radio Resource Control signaling).

#### AI Applications

- AI can enhance handover prediction by analyzing L1 signal quality (RSSI, RSRP, SINR) and L2 scheduling trends.

### 3. Key Protocol Descriptions

#### PDCP (Packet Data Convergence Protocol)

- Functions: Header compression, encryption, integrity protection, reordering, and duplicate detection.

- Important in handover scenarios to prevent packet loss.

#### RLC (Radio Link Control)

- Functions: Segmentation, retransmission, and duplicate removal.

- Modes of Operation:

  - Transparent Mode (TM)

  - Unacknowledged Mode (UM)

  - Acknowledged Mode (AM)

Difference from LTE:
In 5G NR, RLC does not support in-sequence delivery to higher layers to reduce latency and improve flexibility in multi-connectivity.

#### RLC Segmentation

- It. divides large SDUs into smaller PDUs based on available transmission resources.

- It is essential for handling dynamic transport block (TB) sizes.

#### MAC (Medium Access Control)

- Handles logical channel multiplexing/demultiplexing , HARQ retransmissions, and scheduling.

- Logical Channels:

  - BCCH – Broadcast Control Channel for system information.

  - PCCH – Paging Control Channel for paging messages.

  - DCCH – Dedicated Control Channel for device-specific configuration.

  - DTCH – Dedicated Traffic Channel for user data.

- Transport Channels (e.g., DL-SCH, UL-SCH) define how data is transmitted over the radio interface using transport blocks.

### 4. Physical Layer (PHY)

- Functions: Coding, modulation, and mapping to time-frequency resources.

- Physical Channels provide L1/L2 control information:

  - DCI (Downlink Control Information) – Informs devices about downlink and uplink data transmissions.

  - UCI (Uplink Control Information) – Provides scheduling and HARQ feedback to the base station.

### 5. RRC (Radio Resource Control) States

The RRC protocol manages radio resource control between the UE and gNB. It operates in different states:

| State | Description |
| --- | --- |
| RRC_IDLE | No active connection. The device periodically wakes up to receive paging messages. |
| RRC_INACTIVE | Reduced signaling for faster transition to connected state. |
| RRC_CONNECTED | Active connection with established RRC context for data transfer and mobility management. |

#### RRC Functions

- Broadcast of system information.

- Paging for incoming connections.

- Mobility management (e.g., handover, cell reselection).

### 6. AMF (Access and Mobility Management Function)

The AMF is a core network function responsible for control plane signaling and mobility management. Key roles include:

- Authentication: Coordinates with the UDM for user authentication.

- Mobility Management: Handles handover, paging, and UE state transitions.

- Service Request Forwarding: Interacts with the SMF for session management.

Interfaces:

- N1: AMF to UE for signaling.

- N2: AMF to gNB for RAN signaling.

- N11: AMF to SMF for session control.

### 7. Downlink Data Flow Example

- Data flows through the stack starting from the core network (UPF).

- The gNB allocates resources for transmission.

- The PHY layer transmits data over the air, and higher layers (MAC, RLC, PDCP) handle error correction, segmentation, reordering, and decryption before delivering data to the application.