Progress Report Summary – 5G NR Protocol Architecture

souja450

Radio Protocol Architecture

The Radio Access Network (RAN) protocol architecture in cellular networks (such  as 5G, LTE, NR) is a structural layered model of how data is transmitted,  received, and processed.

1. User Plane

The User Plane is used to carry user traffic  (for example, video, voice, browsing traffic) between the User Equipment (UE) and  gNB. It entails the following main protocols and layers:

• SDAP: It manages Quality of Service  (QoS).

• PDCP: It performs functions like IP header compression, ciphering, and  packet reordering.

• RLC: It performs the function of splitting data and requesting transmission of missing  data.

• MAC: It Handles logical channel multiplexing and HARQ retransmissions.

•  PHY: It manages modulation of the signal and its transmission.

Security & Reliability Focus

• The  encryption is done through PDCP ciphering.

• The packet reordering is performed at the  PDCP layer to enhance the reliability.

AI Applications

• In mobility environments, it can optimize packet  reordering, HARQ retransmissions, and duplicate detection to minimize packet loss.

2. Control  Plane

The Control Plane establishes the signaling between the UE and gNB to control network operations such as  authentication, mobility, and security. The main protocols are:

• RRC (Radio Resource Control):  It controls the establishment of the connection, the management of the bearers, and the paging.

•  NAS (Non-Access Stratum): It handles the signaling with the core network.

•  PDCP, RLC, MAC, and PHY layers: They enable the transmission of control information.

Security  Focus

• The encryption and integrity protection are applied to RRC (Radio Resource Control signaling).

AI  Applications

• It can also improve the handover prediction by looking at L1 signal quality  (RSSI, RSRP, SINR) and L2 scheduling trends.

3. Key Protocol  Descriptions

PDCP (Packet Data Convergence Protocol)

• Functions: It provides header compression,  encryption, integrity protection, reordering, and detection of duplicate packets.

• Critical in handover scenarios  to avoid packet loss.

RLC (Radio Link Control)

• Functions: It provides segmentation,  retransmission, and duplicate removal.

• Modes of Operation:

o Transparent Mode (TM)

o  Unacknowledged Mode (UM)

o Acknowledged Mode (AM)

Difference from LTE:

5G NR does not include in-sequence delivery to higher layers at RLC to reduce latency and enhance  adaptability in multi-connectivity.

RLC Segmentation

• It. partitions the SDUs to  PDUs according to the available transmission resources.

• It is required for handling of variable length transport blocks  (TBs).

MAC (Medium Access Control)

• Controls the logical channel mapping/de-mapping,  HARQ retransmission, and scheduling.

• Logical Channels:

o BCCH – System information transmission  control channel (Broadcast Control Channel).

o PCCH – Paging Control Channel for paging messages.

o  DCCH – Device specific configuration information is transmitted through this channel (Dedicated Control Channel).

o  DTCH – It is used for transferring user data (Dedicated Traffic Channel).

• Transport Channels  (e.g., DL-SCH, UL-SCH) define how data is transmitted over the air interface  by means of transport blocks.

4. Physical Layer (PHY)

• Functions: It performs the functions  of coding, modulation, and mapping of bits to time-frequency resources.

• Physical Channels carry the  L1/L2 control information:

o DCI (Downlink Control Information): This informs the devices about  the downlink and uplink data transmissions.

o UCI (Uplink Control Information): This  layer provides the scheduling and HARQ feedback to the base station.

5. RRC (Radio Resource  Control) States

The RRC protocol controls the radio resource control between the UE and gNB.  It works in the following states:

State	Description

RRC_IDLE	There is no active connection.  The device wakes up periodically to listen for paging messages.

RRC_INACTIVE	Reduced signaling for  faster transition to connected state.

RRC_CONNECTED	Established RRC context for data transfer and mobility  management and active connection.

RRC Functions

• Distribution of system information.

• Paging, for incoming  connections.

• Mobility management (e.g., handover, cell reselection).

6. AMF  (Access and Mobility Management Function)

The AMF is a core network function that performs control plane signaling  and mobility management. The main roles are:

• Authentication: Interworks with the UDM for user  authentication.

• Mobility Management: Controls handover, paging, and UE state transition.

• Service Request  Forwarding: It convers with the SMF for session management.

Interfaces:

• N1: UE  to AMF for signaling.

• N2: gNB to AMF for RAN signaling.

• N11: AMF to SMF for session control.

7. An example of the  downlink data path:

• Data flows through the stack from the core network (UPF) up to  the gNB.

• The gNB assigns resources for transmission.

• The PHY layer sends data on  the air, and the remaining layers (MAC, RLC, PDCP) deal with error  correction, segmentation, reordering, and decryption before passing the data to the application.